# Cabinet Projection

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

## Definition

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Cabinet Projection is a type of oblique parallel projection in which a three-dimensional object is projected onto a two-dimensional plane using parallel projection lines, while the depth dimension is usually drawn at half of its actual length.

The main idea is:

Drawn depth=12 Actual depth

This reduction makes the object appear more natural than in Cavalier projection.

## Purpose

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Cabinet projection is used to:

- Represent 3D objects on 2D surfaces.

- Show the front face clearly.

- Represent depth without perspective.

- Reduce the visual distortion caused by full-depth projection.

- Create technical illustrations.

- Represent furniture, mechanical parts, and objects.

- Produce simple 3D diagrams.

## Basic Configuration

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Consider a rectangular box with front vertices:

A, B, C, D

and corresponding back vertices:

A', B', C', D'

The depth lines satisfy:

AA'∥BB'∥CC'∥DD'

However, unlike Cavalier projection, the depth edges are shortened.

If the actual depth is D, then:

D'=D2

## Fundamental Elements

### 1. Projection Plane

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The two-dimensional plane onto which the object is projected.

Π

### 2. Original Object

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The three-dimensional object being represented.

Examples include:

- Cube

- Rectangular prism

- Cylinder

- Furniture

- Mechanical component

### 3. Projection Direction

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The direction in which the projection lines travel:

d

For Cabinet projection:

d⟂̸Π

Therefore, it is an oblique projection.

### 4. Projection Lines

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

All projection lines are parallel:

AA'∥BB'∥CC'

### 5. Projected Object

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The resulting two-dimensional representation.

## Main Characteristics

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

| Property | Cabinet Projection |
| --- | --- |
| Projection type | Parallel |
| Subtype | Oblique |
| Projection lines | Parallel |
| Front face | Usually true size |
| Depth | Usually half scale |
| Depth factor | 12 |
| Perspective | No |
| Vanishing point | No |
| Parallelism | Generally preserved |
| Realistic appearance | Better than Cavalier |

## Depth Dimension

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The most important characteristic of Cabinet projection is the reduction of depth.

If the actual depth is:

D=10 cm

then the commonly used Cabinet representation is:

D'=102 D'=5 cm

Therefore:

Cabinet depth=12 actual depth

The reduction helps prevent the object from appearing excessively deep.

## Projection Angle

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Cabinet projection commonly uses an oblique direction such as:

45∘

for the receding/depth lines.

However, 45∘ is a common convention, not the fundamental definition.

The essential characteristics are:

- Projection lines are parallel.

- The projection is oblique.

- Depth is commonly reduced to one-half.

## Mathematical Representation

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

A general parallel projection can be represented by:

P'=P+td

where:

- P = original point

- P' = projected point

- d = projection direction

- t = scalar

For Cabinet projection, the drawing convention applies a depth reduction factor:

k=12

Thus, for a depth dimension D:

D'=kD

and for the standard Cabinet convention:

D'=12D

## Example 1: Rectangular Box

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Suppose a box has:

W=8 H=5 D=6

In Cabinet projection:

Width

W'=8

Height

H'=5

Depth

D'=12(6)=3

Therefore:

W'=8, H'=5, D'=3

The front face retains its dimensions while the depth is reduced.

## Example 2: Depth Edge

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Suppose an actual depth edge has length:

AB=12 cm

Using the standard Cabinet depth scale:

A'B'=12(12) A'B'=6 cm

So the depth is represented at half scale.

## Example 3: Comparing with Cavalier Projection

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Suppose the actual depth is:

D=8 cm

Cavalier Projection

D'=8 cm

Cabinet Projection

D'=12(8)=4 cm

Therefore:

Cavalier=8 cm Cabinet=4 cm

## How to Construct a Cabinet Projection

### Step 1 — Draw the front face

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Draw the front face using its actual dimensions.

For example:

6×4

### Step 2 — Choose the depth direction

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Choose an oblique direction, commonly:

45∘

### Step 3 — Calculate the represented depth

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

If the actual depth is:

D=6

then:

D'=62=3

### Step 4 — Draw the depth lines

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

From the appropriate vertices, draw parallel lines in the selected direction.

Each depth line has the reduced length:

D'=3

### Step 5 — Connect the corresponding vertices

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Connect the endpoints to complete the 3D representation.

## Cabinet Projection of a Cube

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Consider a cube with side length:

a

The front face is drawn as:

a×a

The actual depth is also:

a

But the Cabinet representation uses:

D'=a2

Therefore:

W'=a H'=a D'=a2

For example, for a cube with:

a=10 cm

the drawing uses:

W'=10 cm H'=10 cm D'=5 cm

## Properties

### 1. Projection lines are parallel

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

AA'∥BB'∥CC'

### 2. Depth is reduced

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Under the standard convention:

D'=D2

### 3. Front face can retain its true dimensions

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The front face is normally drawn at full scale.

### 4. No vanishing point

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The projection lines do not converge.

### 5. Parallelism is generally preserved

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Parallel lines remain parallel in the projection.

### 6. Angles are not generally preserved

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Oblique depth lines can change the apparent angles.

### 7. Depth appears less exaggerated

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Reducing depth makes Cabinet projection visually more natural than Cavalier projection.

## Cabinet Projection vs Cavalier Projection

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

| Feature | Cavalier | Cabinet |
| --- | --- | --- |
| Projection | Oblique parallel | Oblique parallel |
| Front face | Usually true size | Usually true size |
| Depth | Full scale | Usually half scale |
| Depth factor | 1 | 12 |
| Appearance | More exaggerated | More natural |
| Parallel lines | Remain parallel | Remain parallel |
| Perspective | No | No |
| Vanishing point | No | No |

Most important distinction

Cavalier: D'=D Cabinet: D'=D2

## Cabinet Projection vs Orthogonal Projection

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

| Feature | Cabinet | Orthogonal |
| --- | --- | --- |
| Projection | Parallel | Parallel |
| Projection direction | Oblique | Perpendicular |
| Projection lines | Parallel | Parallel |
| Front face | Usually true size | Depends on orientation |
| Depth reduction | Commonly 12 | No Cabinet depth convention |
| Type | Oblique | Orthogonal |

The fundamental distinction is:

Cabinet: d⟂̸Π

while:

Orthogonal: d⟂Π

## Cabinet Projection vs Perspective Projection

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

| Feature | Cabinet | Perspective |
| --- | --- | --- |
| Projection lines | Parallel | Converging |
| Vanishing point | No | Yes |
| Depth | Usually half scale | Changes with distance |
| Parallel lines | Generally remain parallel | May converge |
| Realistic appearance | Moderate | High |
| Technical representation | Useful | Less dimensionally direct |

## Relationship to Oblique Projection

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Cabinet projection belongs to the following hierarchy:

Geometric Projection
│
├── Parallel Projection
│   │
│   ├── Orthogonal Projection
│   │
│   └── Oblique Projection
│       │
│       ├── Cavalier Projection
│       └── Cabinet Projection
│
└── Perspective Projection

Therefore:

Cabinet Projection⊂Oblique Projection⊂Parallel Projection

## Applications

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Cabinet projection can be used for:

- Technical illustrations

- Engineering diagrams

- Mechanical drawings

- Furniture illustrations

- Architectural sketches

- Product diagrams

- Geometry textbooks

- Educational diagrams

- Representing boxes and rectangular objects

- Some CAD and drafting applications

## Advantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

- Simple to construct.

- Easy to interpret.

- Front face can retain true dimensions.

- Reduces excessive depth distortion.

- Preserves parallel relationships.

- Does not require vanishing points.

- More visually natural than Cavalier projection.

- Useful for technical illustrations.

## Limitations

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

- Still does not provide realistic perspective.

- Depth is artificially reduced.

- Angles are not generally preserved.

- Not suitable when true perspective appearance is required.

- The standard 12 depth factor is a convention rather than a universal physical rule.

- Objects can still appear somewhat distorted.

## Special Cases

### Standard Cabinet Projection

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

The most common convention is:

k=12

so:

D'=D2

### Different Depth Scale

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

In some applications, a different depth scale may be chosen:

D'=kD

where k is a selected scale factor.

For example:

k=12

is the standard Cabinet convention.

### If the Projection Direction Becomes Perpendicular

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

If:

d⟂Π

the projection becomes orthogonal projection, rather than oblique Cabinet projection.

## Key Points

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

- Cabinet Projection is an oblique parallel projection.

- Its projection lines are parallel.

- The front face is normally drawn at its true size.

- Depth is usually reduced to one-half.

- A 45∘ receding direction is commonly used.

- It has no vanishing point.

- It looks more natural than Cavalier projection.

- It is useful for technical and educational illustrations.

- Its defining practical convention is:

Drawn depth=12 Actual depth

## Conclusion

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [7/25/2026]

Cabinet Projection represents a 3D object on a 2D plane using parallel oblique projection lines, while reducing the depth dimension—commonly to one-half of its actual length. This reduction produces a less exaggerated and more visually balanced representation than Cavalier projection, while retaining the simplicity and parallelism of parallel projection.